OpenFIRE Light Gun & Wireless Dongle
User Manual

Firmware: OpenFIRE-Firmware-ESP32 (ESP32-S3 light gun + ESP-NOW Dongle, English OLED UI)

Optional configuration app: OpenFIRE Desktop App (Team OpenFIRE / OpenFIRE-App-cn)

Document version: 1.0  |  Date: June 2026

Note: OLED text in this manual matches the default English firmware strings (build without -D OLED_MENU_ZH).

# 1. Product Overview

OpenFIRE is a four-point infrared (4IR) light gun system. The ESP32-S3 port supports two PC connection methods: (1) USB wired direct connection; (2) ESP-NOW wireless via a Dongle. In wireless mode the PC sees the same HID device as wired—no extra game or force-feedback adapters are required.

This manual covers: light gun (GUN) daily use, OLED pause menu, calibration, player IDs (P1–P4); wireless receiver (Dongle) and wireless pedal pairing, on-screen status, and troubleshooting.

## 1.1 Package contents (reference)

Light gun (IR camera, buttons, OLED, optional rumble/solenoid, etc.)

USB cable (wired play, charging, or firmware flashing)

Wireless Dongle (ESP32-S3, plugs into PC USB; SSD1306/TFT models show pairing status on OLED)

IR emitter bar(s) (mount on monitor top/bottom for Square layout, or on all four sides for Diamond)

(Optional) Wireless pedal module for Time Crisis–style cover mechanics

## 1.2 Safety

On boot and during auto stick center calibration (~2 s), do not touch the analog stick.

Solenoid recoil produces mechanical impact and heat—never point the gun at people or animals.

Monitor the temperature readout on the OLED during long sessions; overtemperature protection may limit FFB.

Flash firmware or rewire only when powered off. See docs/BOARDS_PIN.md for board pin maps.

# 2. First Boot & Connection

## 2.1 Boot sequence

After power-on the gun runs this sequence automatically:

| Stage | OLED text (English) | Meaning |
| --- | --- | --- |
| Stick auto-cal | CALIBRATION READY | ~2 s—do not move stick |
| USB detection | go in usb mode → wait for usb mount | Wired vs wireless path |
| Link ready | !! LINK READY !! / USB mounted | USB or wireless link up |
| First-time aim cal | Welcome! / Pull trigger / start calibration! | Pull trigger to start 5-point cal |

## 2.2 Wired USB

Connect the gun to the PC with USB.

Wait for Windows to enumerate the HID device (name typically FIRECon P1–P4 depending on Gun ID).

Top bar may show  USB mounted ; lower-left shows USB icon.

Use the OpenFIRE Desktop App on the COM port for advanced settings (Docked mode—see Section 10).

## 2.3 Wireless Dongle (recommended order)

Step 1: Plug the Dongle into the PC USB port. Wait for startup (OLED: Scanning WiFi / best channel eval / Please wait, ~10–15 s).

Step 2: Do NOT plug the gun into PC USB. Power on the gun.

Step 3: When the Dongle shows Ch:xx Pair / Turn on lightgun / ESP-NOW listen, the gun should start pairing broadcast.

Gun OLED top bar may show Ch:xx Waiting link.

Step 4: After success the Dongle shows RF link OK → Player / Channel / Gun MAC; PC shows the gun’s configured device name.

Step 5: With a wireless pedal, the Dongle may show AWAIT PEDAL; the gun shows Search Pedal [xxs] while searching.

Step 6: Wi-Fi icon at lower-left indicates wireless mode.

On subsequent boots the gun prefers the last paired Dongle; if that fails it rescans all channels.

## 2.4 Boy Mode (offline play without PC)

If wireless is down and USB is not mounted, hold the trigger within ~3 s of power-on to enter Boy Mode: OLED shows Boy? BOOT/TRIG Xs countdown, then  BOY MODE READY . Allows local solenoid/rumble testing without a PC (hardware dependent).

# 3. OLED Status Bar (Gameplay Screen)

| Area | OLED example | Meaning |
| --- | --- | --- |
| Top left | P1: P_A Diam 42C | Player ID, profile slot (P_A…P_D), layout Diam/Squa, temperature |
| Top right | OFF / 300 | Play Timer: OFF = disabled; number = remaining seconds |
| Full top row | Prof: ProfileA P1 | Shown when pausing or switching profiles |
| Lower left | USB / Wi-Fi icon | Wired or wireless Dongle |
| Lower center | RF / LOW / AF | Rumble FFB, Low Button mode, Autofire (abbreviations) |
| Lower right | Mouse / Gamepad / MiSTer icon | Current USB output mode |

Transient top-bar messages include: Mode Changed: Gamepad, Autofire: ON, Low Button: OFF, Rumble FFB: ON, Sent Escape Key!, Play time over, etc.

# 4. Pause Menu (English OLED)

The pause menu switches profiles, runs calibration, changes USB mode, sets Gun ID, saves settings, etc. Two interaction styles exist, controlled by the simplePause option (default is usually Direct Pause hotkeys).

## 4.1 Enter / exit pause

| Action | Button combo | OLED hint |
| --- | --- | --- |
| Enter pause | Reload + Select | Using + current profile name |
| Exit pause | Reload + Home | Return to gameplay screen |
| Hold to pause (if Hold to Pause enabled) | Trigger + A ~2.5 s | — |
| Simple Pause hold exit | A + B ~half Hold to Pause time | — |

## 4.2 Runtime shortcuts (no pause menu)

| Function | Button combo | OLED message |
| --- | --- | --- |
| Send Esc to PC | Reload + Start | Sent Escape Key! |
| Send Esc (quick) | Trigger + A + C (release all together) | Sent Escape Key! |
| Mouse/KB ↔ Gamepad | A + B + Trigger | Mode Changed: Mouse/KB / Gamepad |

Note: Gun C maps to the Reload button bit. MiSTer mode is selected via  Mode Change  in Simple Pause.

## 4.3 Direct Pause (default hotkey mode)

In pause, four lines show  A > / B > / Str> / Sel>  with profile names; most actions use combos:

| Function | Button combo |
| --- | --- |
| Screen calibration | Trigger while paused |
| Save all settings to Flash | Start + Select |
| IR sensitivity + | B + Up |
| IR sensitivity − | B + Down |
| Run mode Normal | Start + A |
| Run mode Average / Average2 | Start + B |
| Send Esc | Reload + Start or Trigger+A+C |
| Mouse/KB ↔ Gamepad | A + B + Trigger |
| Switch profile A/B/Start/Select | Press matching face button on pause screen |
| Skip center step during cal | Press A during calibration |

## 4.4 Simple Pause (scrolling list mode)

If Simple Pause is enabled in the OpenFIRE App: A = previous item, B = next item, Trigger = confirm. Menu labels match firmware exactly:

| OLED label (English) | Function |
| --- | --- |
| Center Calibrate | Set current stick position as new center (requires analog pins) |
| Range Calibrate | ~4 s stick rotation + release to center (RangeCal: rotate → Release to center → OK) |
| Calibrate | Enter 5-point screen aim calibration |
| Profile Select | Pick Profile A/B/Start/Select (sub-screen: A>/B>/Str>/Sel>) |
| Mode Change | Cycle Mouse/KB → Gamepad → MiSTer ( Current: … ) |
| Layout Toggle | Square ↔ Diamond ( Layout: Square / Diamond ) |
| Gun ID / Gun ID (P1-P4) | Set player P1–P4; updates USB PID and device name |
| Stick Output Mode | Stick: Gamepad / D-Pad / Keyboard Arrows |
| Deadzone | Stick deadzone 0–30% ( Deadzone: N% ) |
| Axis Mode | Signed / Unsigned axis |
| Swap Sticks | Swap IR aim axis with physical stick L/R |
| Play Timer | 0 / 5 / 10 / 15 / 20 min ( Play Timer: OFF / Play Timer: N min ); expiry: Play time over |
| Save Gun Settings | Write profiles & globals to Flash ( Saving... / Save successful ) |
| Rumble Toggle | Master rumble motor ( Toggling Rumble ON/OFF ) |
| Solenoid Toggle | Master solenoid ( Toggling Solenoid ON/OFF ) |
| Autofire Toggle | Autofire ( Autofire: ON/OFF ) |
| Low Button Toggle | Low-button mapping ( Low Button: ON/OFF ) |
| Rumble FFB Toggle | Use rumble instead of solenoid for serial FFB ( Rumble FFB: ON/OFF/N/A ) |
| Analog Invert X / Y | Invert stick axis |
| Send Escape Keypress | Send one Esc key to PC |

Items without hardware are skipped automatically. Gun ID sub-menu: A/B pick P1–P4, Trigger saves, Reload+Home cancels.

# 5. Gun ID & Multiple Guns (P1–P4)

Each gun can be P1, P2, P3, or P4 (USB Product ID 1–4, device name e.g. FIRECon P1).

Set via OLED  Gun ID: Pn , Simple Pause  Gun ID , or App → Settings → USB Identity.

After changing Gun ID you must  Save Gun Settings  and replug USB for Windows to see the new ID.

Multi-gun: assign unique IDs; use separate USB ports or one Dongle per gun.

# 6. USB Output Modes

| Mode | OLED | Typical use |
| --- | --- | --- |
| Mouse/KB (default) | Current: Mouse/KB | MAME, Demul, TeknoParrot, etc. |
| Gamepad | Current: Gamepad | Games expecting gamepad HID |
| MiSTer | Current: MiSTer | MiSTer FPGA platform |

Switch via Simple Pause  Mode Change , runtime A+B+Trigger (Mouse/KB ↔ Gamepad), or Desktop App.

# 7. Calibration Profiles

Four built-in profiles: Profile A, Profile B, Profile Start, Profile Select.

Idle bar shows P_A…P_D shorthand; profile picker may show  Name (Square)  or  Name (Diamond) .

Direct Pause: press the button matching A>/B>/Str>/Sel> on the pause screen.

Simple Pause:  Profile Select  submenu.

Desktop App: Profiles tab → Confirm to save.

Always  Save Gun Settings  or Start+Select before power-off.

# 8. Calibration Guide

## 8.1 Screen aim calibration (required)

Maps IR coordinates to your monitor. Required on first use or after changing displays.

Start: pull trigger at first boot; pause  Calibrate ; Direct Pause trigger; App cal window.

Top bar during cal:  Cali:  + profile name.

Steps: Top → Bottom → Left → Right → Center → Verify. Reload+Home cancels and restores old data.

## 8.2 Analog stick calibration

| Type | OLED menu / message | Action |
| --- | --- | --- |
| Boot auto center | CALIBRATION READY | ~2 s at every boot—don’t touch stick |
| Center Calibrate | Center calibrated | Set current position as center |
| Range Calibrate | RangeCal: rotate / OK | Rotate stick ~4 s, release to center |

Stick range calibration is done on the gun OLED ( Range Calibrate ); the Desktop App has no range-cal UI yet.

## 8.3 IR emitter mounting

Square layout (Squa): emitters on top and bottom of the monitor.

Diamond layout (Diam): emitters at the mid-point of all four sides.

Use the App’s Open IR Emitter Alignment Assistant for a layout diagram.

# 9. Wireless Dongle

## 9.1 What the Dongle does

The Dongle is an ESP32-S3 receiver on the PC. It talks to the gun over ESP-NOW (2.4 GHz) and presents the same USB identity and forwards HID plus serial force-feedback data.

## 9.2 OLED status (English)

| OLED text | Meaning | What to do |
| --- | --- | --- |
| OpenFIRE DONGLE | Boot logo | Wait |
| RF sniff ch / Scanning WiFi | Picking least-congested channel | Leave USB plugged ~10–15 s |
| Best channel eval / Please wait | Channel survey | Wait |
| Ch:xx Pair / Turn on lightgun / ESP-NOW listen | Waiting for gun | Power gun (no PC USB on gun) |
| RF link OK / Wait gun stream | RF up, waiting HID stream | Wait a few seconds |
| AWAIT PEDAL | Waiting for wireless pedal | Power pedal if used; else wait |
| Player: N / Channel: N / Gun: MAC | Paired | Test on PC |

TFT Dongles (e.g. LilyGO) may show Searching / best channel / Scanning WiFi / Connecting / AWAIT PEDAL / Player:N.

## 9.3 Wireless pedal

The pedal has no OLED. While searching, the gun top bar shows Search Pedal [xxs].

After the gun pairs, the Dongle may show AWAIT PEDAL until the pedal handshake completes.

Pedal gameplay matches a wired pedal; status is LED-only on the pedal unit.

## 9.4 PC USB & COM port

Before pairing completes the PC may see a placeholder device; afterward the gun’s configured name appears.

Force feedback (Mamehooker, etc.): select the Dongle COM port at 9600 baud.

Change VID/PID/mode on the gun, then re-pair so the Dongle refreshes its USB descriptor.

## 9.5 Multi-device tips

Recommended: one gun + one Dongle. Multiplayer: unique Gun IDs and separate Dongles.

Avoid multiple Dongles in pairing mode competing for the same gun.

If a gun sticks to an old Dongle MAC, clear binding or wait for timeout and full-channel rescan.

# 10. OpenFIRE Desktop App (optional)

Pin mapping, toggles, profile parameters, USB P1–P4, IR calibration, button test, FFB test.

Connect via COM port; App sends Dock command to enter Docked mode and sync data.

Click Confirm after edits. Gun ID: Settings → USB Identity.

Use a firmware build compatible with your App branch (OpenFIRE-App or OpenFIRE-App-cn).

# 11. Force Feedback & MAMEHOOKER

With MAMEHOOKER enabled at compile time, gun/Dongle expose a 9600 baud serial channel.

PC FFB software → serial → (wireless: via Dongle) → gun drives solenoid/rumble.

OLED may show Mamehook ammo/life when configured.

Rumble FFB Toggle  chooses rumble motor vs solenoid for serial FFB (separate from  Rumble Toggle ).

Solenoid Toggle  = solenoid hardware;  Rumble Toggle  = rumble motor hardware.

# 12. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Dongle stuck on Turn on lightgun / Searching | Gun off, gun on PC USB, out of range | Dongle first, then gun; unplug gun USB; move closer |
| Gun keeps Search Pedal | Pedal off or out of range | Power pedal; move near Dongle |
| PC name stays P1 | Gun ID not saved or USB not replugged | Save Gun Settings; replug USB |
| App cannot set Gun ID | App/firmware protocol mismatch | Update to matching firmware/App pair |
| Wireless drops after connect | 2.4 GHz congestion | Reduce distance; fewer Wi-Fi devices |
| OLED blank | Wrong I2C address/pins for board | Verify PlatformIO board env; replug USB |
| Aim offset | Uncalibrated or new monitor | Re-run screen cal; check Diam/Squa vs emitter mount |
| FFB dead | Wrong COM or FFB disabled | Dongle COM @ 9600; enable Rumble FFB Toggle; check wiring |
| Locked after Play time over | Play Timer expired | Wait for lockout to end or set Play Timer: OFF |
| Stick drift on boot | Stick touched during cal | Don’t touch stick first 2 s; run Center/Range Calibrate |

# Appendix A: Quick daily workflow

[Wired] USB to PC → confirm USB icon → play.

[Wireless] Dongle to PC → gun on (no USB to gun) → Wi-Fi icon → play.

[Settings/cal] Reload+Select → pause → change → Save Gun Settings or Start+Select → Reload+Home.

[Esc] Reload+Start or Trigger+A+C.

[Multiplayer] Unique Gun ID per gun → save → replug USB / one Dongle each.

# Appendix B: Chinese vs English firmware (menu cross-reference)

| Chinese (OLED_MENU_ZH) | English (default) |
| --- | --- |
| 中心校准 | Center Calibrate |
| 摇杆校准 | Range Calibrate |
| 屏幕校准 | Calibrate |
| 配置选择 | Profile Select |
| 少键模式 | Low Button Toggle |
| 震动FFB开关 | Rumble FFB Toggle |
| 游戏计时 | Play Timer |
| 玩具模式 | Boy Mode |
| 枪号 | Gun ID |
| 布局 ST / SC | Diam / Squa |

# Appendix C: Supported boards (ESP32-S3)

ESP32-S3 WROOM-1 DevKitC-1 (N16R8 / N8R2)

Waveshare ESP32-S3-Pico / ESP32-S3-Zero (N8R8 / N4R2)

ESP32-S3 Super Mini (4 MB) and ESP32-S3 N8R8 (8 MB Flash + 8 MB PSRAM)

See docs/BOARDS_PIN.md and lightgun/platformio.ini environments for build targets.

# Appendix D: Resources & license

Firmware: OpenFIRE-Firmware-ESP32 (github.com/wei134102/OpenFIRE-Firmware-ESP32)

Upstream: Team OpenFIRE / alessandro-satanassi OpenFIRE-Firmware-ESP32

Board pinouts: docs/BOARDS_PIN.md

OpenFIRE is open source; ESP32 port, ESP-NOW wireless, and OLED i18n by community maintainers.

Features depend on your flashed firmware version and hardware configuration.